BGP

Andrew Tang

Purpose

The purpose of this lab is to understand how different routing protocols interact in a network where there are multiple protocols and how BGP is used to exchange routes between separate routing domains.

Background Information

Networks now rarely rely on a single routing technology. Large organizations often divide their networks into separate departments or regions, all of which may use a different method, or routing protocol, that determines how data should travel from one location to another. The challenge presented in this lab is how these different routing systems communicate with each other in a controlled and reliable way.

Some routing protocols are designed to work inside a single organization. These are known as Interior Gateway Protocols (IGPs). In this lab, two IGPs are used: OSPF (Open Shortest Path First) and EIGRP (Enhanced Interior Gateway Routing Protocol). OSPF calculates the fastest path through a network based on link cost, while EIGRP chooses paths using a combination of speed reliability and delay. Each of these protocols works very well within its own area, but they are not designed to communicate directly with each other. This limitation allows large networks to be divided for performance, security, and administrative reasons.

When a network becomes large enough, it is treated as an Autonomous System (AS). An autonomous system is a group of routers under a single administrative control that follows a common routing policy. OSPF and EIGRP typically operate within an autonomous system. To connect different autonomous systems, networks use Border Gateway Protocol (BGP). BGP is the routing protocol that makes the modern internet possible. Rather than focusing on the fastest path, BGP focuses on policy and control, allowing organizations to decide which routes are shared and how traffic enters or exits their network. In this lab, BGP acts as a translator between the OSPF network and the EIGRP network. It allows routes learned in one routing system to be advertised to another without requiring them to run the same protocol. Unlike simpler routing systems that only focus on finding the fastest path, BGP exchanges information in structured “packages” of routing data. These packages contain not just the destination network, but also context about where the route came from and how it should be treated. This design allows organizations to make careful, policy-based decisions about how data moves between large networks. A BGP package (or BGP route advertisement) is a message sent from one router to another that contains 1) Destination network: the address range that can be reached, 2) Path information: a record of which administrative networks the route has passed through, 3) Next-hop information: Where traffic should be sent next to reach the destination, and 4) Route origin details: how the route entered BGP. Route origin, as used in this lab, makes it so that routes from OSPF and EIGRP appear with a different origin indicator. This helps network administrators understand the trust level and source of each route.

Lab Summary

In this lab, a multi-protocol network was configured to show how routing information can be exchanged between different routing regions using BGP. The topology is made up of three separate routing regions running OSPF, EIGRP, and IS-IS, with BGP being used as the intermediary protocol to allow for communication between them. The left side of the topology was OSPF, the middle EIGRP, and the right side IS-IS. Two edge routers were configured with eBGP to form BGP peering between the two routing domains.

Routes that were learned through OSPF, EIGRP, and IS-IS were redistributed into the BGP, which allowed each routing domain to advertise its networks across the connection. BGP routers were learned and then distributed back into the routing domain for full end-to-end reachability across the entire network. This required careful configuration to ensure that routes were shared in both directions and that loops were avoided.

At the end of the lab, all routers learned routes from the other routing domains, and end-to-end connectivity across the entire network was achieved.

Lab Commands

router eigrp <AS-number>

Starts the EIGRP routing process

- Identifies the routing domain

- Has to match on all EIGRP routers

no auto-summary

Prevents incorrect route advertisement

router bgp <AS-number>

Starts the BGP routing process

- Each edge router used a different AS number

- Simulates independent networks

neighbor <IP> remote-as <AS-number>

Defines a BGP neighbor

- Establishes peering

- AS number must match the neighbor’s BGP AS

redistribute eigrp <AS-number>

Imports BGP routes into BGP, makes EIGRP network visible to others

redistribute bgp <AS-number>

Puts BGP routes into an IGP

- Allows internal routers to learn external networks

metric <value>

Defines path cost for redistributed routes

- EIGRP requires a metric to evaluate routes

- Prevents routes from being ignored

show ip bgp

Displays the BGP table

- Shows BGP-learned routes

- Displays AS paths and origin codes

- Helps verify BGP updates

show ip cef

Checks the forwarding table

- Confirms whether traffic can be forwarded

- Explains why there may be routes but pings fail

show ip protocols

Summarizes the routing protocol configuration

- Displays redistribution status

- Shows which networks are being advertised

Network Diagram

Configurations

R1#show ip route

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2

ia - IS-IS inter area, * - candidate default, U - per-user static route

o - ODR, P - periodic downloaded static route, H - NHRP, l - LISP

a - application route

+ - replicated route, % - next hop override, p - overrides from PfR

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 6 subnets, 2 masks

C        10.0.0.0/30 is directly connected, GigabitEthernet0/0/0

L        10.0.0.1/32 is directly connected, GigabitEthernet0/0/0

O        10.0.1.0/30 [110/2] via 10.0.0.2, 01:18:24, GigabitEthernet0/0/0

O E2     10.0.2.0/30 [110/20] via 10.0.0.2, 00:53:47, GigabitEthernet0/0/0

O E2     10.0.3.0/30 [110/20] via 10.0.0.2, 00:53:47, GigabitEthernet0/0/0

O E2     10.0.4.0/30 [110/20] via 10.0.0.2, 00:53:16, GigabitEthernet0/0/0

R1#show ipv6 route

IPv6 Routing Table - default - 7 entries

Codes: C - Connected, L - Local, S - Static, U - Per-user Static route

B - BGP, R - RIP, I1 - ISIS L1, I2 - ISIS L2

IA - ISIS interarea, IS - ISIS summary, D - EIGRP, EX - EIGRP external

ND - ND Default, NDp - ND Prefix, DCE - Destination, NDr - Redirect

O - OSPF Intra, OI - OSPF Inter, OE1 - OSPF ext 1, OE2 - OSPF ext 2

ON1 - OSPF NSSA ext 1, ON2 - OSPF NSSA ext 2, a - Application

C   2000::/64 [0/0]

via GigabitEthernet0/0/0, directly connected

L   2000::1/128 [0/0]

via GigabitEthernet0/0/0, receive

O   2001::/64 [110/2]

via FE80::2C1:B1FF:FEB3:8C01, GigabitEthernet0/0/0

OE2 2002::/64 [110/1]

via FE80::2C1:B1FF:FEB3:8C01, GigabitEthernet0/0/0

OE2 2003::/64 [110/1]

via FE80::2C1:B1FF:FEB3:8C01, GigabitEthernet0/0/0

OE2 2004::/64 [110/1]

via FE80::2C1:B1FF:FEB3:8C01, GigabitEthernet0/0/0

L   FF00::/8 [0/0]

via Null0, receive

R2#show ip route

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2

ia - IS-IS inter area, * - candidate default, U - per-user static route

o - ODR, P - periodic downloaded static route, H - NHRP, l - LISP

a - application route

+ - replicated route, % - next hop override, p - overrides from PfR

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 7 subnets, 2 masks

C        10.0.0.0/30 is directly connected, GigabitEthernet0/0/1

L        10.0.0.2/32 is directly connected, GigabitEthernet0/0/1

C        10.0.1.0/30 is directly connected, GigabitEthernet0/0/0

L        10.0.1.1/32 is directly connected, GigabitEthernet0/0/0

B        10.0.2.0/30 [20/0] via 10.0.1.2, 00:54:47

B        10.0.3.0/30 [20/3072] via 10.0.1.2, 00:54:47

B        10.0.4.0/30 [20/258816] via 10.0.1.2, 00:54:16

R2#show ipv6 route

IPv6 Routing Table - default - 8 entries

Codes: C - Connected, L - Local, S - Static, U - Per-user Static route

B - BGP, R - RIP, I1 - ISIS L1, I2 - ISIS L2

IA - ISIS interarea, IS - ISIS summary, D - EIGRP, EX - EIGRP external

ND - ND Default, NDp - ND Prefix, DCE - Destination, NDr - Redirect

O - OSPF Intra, OI - OSPF Inter, OE1 - OSPF ext 1, OE2 - OSPF ext 2

ON1 - OSPF NSSA ext 1, ON2 - OSPF NSSA ext 2, a - Application

C   2000::/64 [0/0]

via GigabitEthernet0/0/1, directly connected

L   2000::2/128 [0/0]

via GigabitEthernet0/0/1, receive

C   2001::/64 [0/0]

via GigabitEthernet0/0/0, directly connected

L   2001::1/128 [0/0]

via GigabitEthernet0/0/0, receive

B   2002::/64 [20/0]

via FE80::521C:B0FF:FE63:37A1, GigabitEthernet0/0/0

B   2003::/64 [20/3072]

via FE80::521C:B0FF:FE63:37A1, GigabitEthernet0/0/0

B   2004::/64 [20/258816]

via FE80::521C:B0FF:FE63:37A1, GigabitEthernet0/0/0

L   FF00::/8 [0/0]

via Null0, receive

R3#show ip route

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2

ia - IS-IS inter area, * - candidate default, U - per-user static route

o - ODR, P - periodic downloaded static route, H - NHRP, l - LISP

a - application route

+ - replicated route, % - next hop override, p - overrides from PfR

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 7 subnets, 2 masks

B        10.0.0.0/30 [20/0] via 10.0.1.1, 01:19:49

C        10.0.1.0/30 is directly connected, GigabitEthernet0/0/1

L        10.0.1.2/32 is directly connected, GigabitEthernet0/0/1

C        10.0.2.0/30 is directly connected, GigabitEthernet0/0/0

L        10.0.2.1/32 is directly connected, GigabitEthernet0/0/0

D        10.0.3.0/30 [90/3072] via 10.0.2.2, 00:55:22, GigabitEthernet0/0/0

D EX     10.0.4.0/30 [170/258816] via 10.0.2.2, 00:55:12, GigabitEthernet0/0/0

R3#show ipv6 route

IPv6 Routing Table - default - 8 entries

Codes: C - Connected, L - Local, S - Static, U - Per-user Static route

B - BGP, R - RIP, I1 - ISIS L1, I2 - ISIS L2

IA - ISIS interarea, IS - ISIS summary, D - EIGRP, EX - EIGRP external

ND - ND Default, NDp - ND Prefix, DCE - Destination, NDr - Redirect

O - OSPF Intra, OI - OSPF Inter, OE1 - OSPF ext 1, OE2 - OSPF ext 2

ON1 - OSPF NSSA ext 1, ON2 - OSPF NSSA ext 2, a - Application

B   2000::/64 [20/0]

via FE80::2C1:B1FF:FEB3:8C00, GigabitEthernet0/0/1

C   2001::/64 [0/0]

via GigabitEthernet0/0/1, directly connected

L   2001::2/128 [0/0]

via GigabitEthernet0/0/1, receive

C   2002::/64 [0/0]

via GigabitEthernet0/0/0, directly connected

L   2002::1/128 [0/0]

via GigabitEthernet0/0/0, receive

D   2003::/64 [90/3072]

via FE80::267E:12FF:FE4D:E7B1, GigabitEthernet0/0/0

EX  2004::/64 [170/258816]

via FE80::267E:12FF:FE4D:E7B1, GigabitEthernet0/0/0

L   FF00::/8 [0/0]

via Null0, receive

R4#show ip route

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2

ia - IS-IS inter area, * - candidate default, U - per-user static route

o - ODR, P - periodic downloaded static route, H - NHRP, l - LISP

a - application route

+ - replicated route, % - next hop override, p - overrides from PfR

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 7 subnets, 2 masks

D EX     10.0.0.0/30 [170/258816] via 10.0.2.1, 00:55:31, GigabitEthernet0/0/1

D        10.0.1.0/30 [90/3072] via 10.0.2.1, 00:55:31, GigabitEthernet0/0/1

C        10.0.2.0/30 is directly connected, GigabitEthernet0/0/1

L        10.0.2.2/32 is directly connected, GigabitEthernet0/0/1

C        10.0.3.0/30 is directly connected, GigabitEthernet0/0/0

L        10.0.3.1/32 is directly connected, GigabitEthernet0/0/0

B        10.0.4.0/30 [20/0] via 10.0.3.2, 00:55:04

R4#show ipv6 route

IPv6 Routing Table - default - 8 entries

Codes: C - Connected, L - Local, S - Static, U - Per-user Static route

B - BGP, R - RIP, I1 - ISIS L1, I2 - ISIS L2

IA - ISIS interarea, IS - ISIS summary, D - EIGRP, EX - EIGRP external

ND - ND Default, NDp - ND Prefix, DCE - Destination, NDr - Redirect

O - OSPF Intra, OI - OSPF Inter, OE1 - OSPF ext 1, OE2 - OSPF ext 2

ON1 - OSPF NSSA ext 1, ON2 - OSPF NSSA ext 2, a - Application

EX  2000::/64 [170/258816]

via FE80::521C:B0FF:FE63:37A0, GigabitEthernet0/0/1

D   2001::/64 [90/3072]

via FE80::521C:B0FF:FE63:37A0, GigabitEthernet0/0/1

C   2002::/64 [0/0]

via GigabitEthernet0/0/1, directly connected

L   2002::2/128 [0/0]

via GigabitEthernet0/0/1, receive

C   2003::/64 [0/0]

via GigabitEthernet0/0/0, directly connected

L   2003::1/128 [0/0]

via GigabitEthernet0/0/0, receive

B   2004::/64 [20/0]

via FE80::CE7F:76FF:FE6A:A621, GigabitEthernet0/0/0

L   FF00::/8 [0/0]

via Null0, receive

R5#show ip route

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2

ia - IS-IS inter area, * - candidate default, U - per-user static route

o - ODR, P - periodic downloaded static route, H - NHRP, l - LISP

a - application route

+ - replicated route, % - next hop override, p - overrides from PfR

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 7 subnets, 2 masks

B        10.0.0.0/30 [20/258816] via 10.0.3.1, 00:54:29

B        10.0.1.0/30 [20/3072] via 10.0.3.1, 00:54:29

B        10.0.2.0/30 [20/0] via 10.0.3.1, 00:54:29

C        10.0.3.0/30 is directly connected, GigabitEthernet0/0/1

L        10.0.3.2/32 is directly connected, GigabitEthernet0/0/1

C        10.0.4.0/30 is directly connected, GigabitEthernet0/0/0

L        10.0.4.1/32 is directly connected, GigabitEthernet0/0/0

R5#show ipv6 route

IPv6 Routing Table - default - 8 entries

Codes: C - Connected, L - Local, S - Static, U - Per-user Static route

B - BGP, R - RIP, I1 - ISIS L1, I2 - ISIS L2

IA - ISIS interarea, IS - ISIS summary, D - EIGRP, EX - EIGRP external

ND - ND Default, NDp - ND Prefix, DCE - Destination, NDr - Redirect

O - OSPF Intra, OI - OSPF Inter, OE1 - OSPF ext 1, OE2 - OSPF ext 2

ON1 - OSPF NSSA ext 1, ON2 - OSPF NSSA ext 2, a - Application

B   2000::/64 [20/258816]

via FE80::267E:12FF:FE4D:E7B0, GigabitEthernet0/0/1

B   2001::/64 [20/3072]

via FE80::267E:12FF:FE4D:E7B0, GigabitEthernet0/0/1

B   2002::/64 [20/0]

via FE80::267E:12FF:FE4D:E7B0, GigabitEthernet0/0/1

C   2003::/64 [0/0]

via GigabitEthernet0/0/1, directly connected

L   2003::2/128 [0/0]

via GigabitEthernet0/0/1, receive

C   2004::/64 [0/0]

via GigabitEthernet0/0/0, directly connected

L   2004::1/128 [0/0]

via GigabitEthernet0/0/0, receive

L   FF00::/8 [0/0]

via Null0, receive

R6#show ip route

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2

ia - IS-IS inter area, * - candidate default, U - per-user static route

o - ODR, P - periodic downloaded static route, H - NHRP, l - LISP

a - application route

+ - replicated route, % - next hop override, p - overrides from PfR

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 6 subnets, 2 masks

i L2     10.0.0.0/30 [115/10] via 10.0.4.1, 00:53:36, GigabitEthernet0/0/1

i L2     10.0.1.0/30 [115/10] via 10.0.4.1, 00:53:36, GigabitEthernet0/0/1

i L2     10.0.2.0/30 [115/10] via 10.0.4.1, 00:53:36, GigabitEthernet0/0/1

i L2     10.0.3.0/30 [115/20] via 10.0.4.1, 00:53:46, GigabitEthernet0/0/1

C        10.0.4.0/30 is directly connected, GigabitEthernet0/0/1

L        10.0.4.2/32 is directly connected, GigabitEthernet0/0/1

R6#show ipv6 route

IPv6 Routing Table - default - 7 entries

Codes: C - Connected, L - Local, S - Static, U - Per-user Static route

B - BGP, R - RIP, I1 - ISIS L1, I2 - ISIS L2

IA - ISIS interarea, IS - ISIS summary, D - EIGRP, EX - EIGRP external

ND - ND Default, NDp - ND Prefix, DCE - Destination, NDr - Redirect

O - OSPF Intra, OI - OSPF Inter, OE1 - OSPF ext 1, OE2 - OSPF ext 2

ON1 - OSPF NSSA ext 1, ON2 - OSPF NSSA ext 2, a - Application

I2  2000::/64 [115/10]

via FE80::CE7F:76FF:FE6A:A620, GigabitEthernet0/0/1

I2  2001::/64 [115/10]

via FE80::CE7F:76FF:FE6A:A620, GigabitEthernet0/0/1

I2  2002::/64 [115/10]

via FE80::CE7F:76FF:FE6A:A620, GigabitEthernet0/0/1

I2  2003::/64 [115/20]

via FE80::CE7F:76FF:FE6A:A620, GigabitEthernet0/0/1

C   2004::/64 [0/0]

via GigabitEthernet0/0/1, directly connected

L   2004::2/128 [0/0]

via GigabitEthernet0/0/1, receive

L   FF00::/8 [0/0]

via Null0, receive

Problems

We encountered several issues that prevented the connectivity of the network, even when routing protocols appeared to be configured correctly. These issues emphasized the complexity of multi-protocol routing environments.

One of the primary issues was that routes appeared in the routing tables, but connectivity across the network still failed. Routes were successfully learned through OSPF, EIGRP, IS-IS, and BGP, which initially told us that the network was functioning correctly. However, when we tried to ping between routers in different protocols, it was unsuccessful. This  taught me that the presence of routes in the routing table does not guarantee the forwarding of traffic. When we looked deeper, we saw problems related to next-hop reachability. On certain routers, BGP routes pointed to next-hop addresses that were not reachable within the local routing domain, which caused routes to be unusable. The issue was identified when we ran the show ip cef command, which showed that forwarding entries for specific destinations did not exist, even though routes were listed in the routing table.

Another one of our challenges was route distribution behavior. Some routers displayed multiple routing entries for the same network, which made it difficult to determine which route the router was actually using. When we analyzed the route codes (B, D, L) and administrative distances, we saw how routers prioritized routes learned from different protocols.

Additionally, BGP origin codes were confusing at first. Routes redistributed into BGP showed an origin of ?, which made us question whether the routes were misconfigured. Once we troubleshooted and verified it, we learned that this behavior is expected when routes are imported from other routing protocols.

From this lab, I learned that troubleshooting routing issues require more than verifying configurations. Commands like show ip cef are essential for confirming that routes are usable for forwarding, not just in the routing table. This lab showed the importance of validating next-hop reachability and knowing how routers make forwarding decisions.

Conclusion

This lab explored how multiple different routing protocols can be integrated into a single network using BGP as an intermediary. We configured a network containing separate OSPF, EIGRP, and IS-IS routing domains, and implemented BGP between edge routers to enable the protocols to exchange routes, which they would otherwise be incompatible.

Routing protocols were successfully configured and routes between OSPF, EIGRP, IS-IS, and BGP were redistributed to allow full visibility of destinations across the network. Initially, routing information appeared correct in routing tables, but connectivity issues occurred due to next-hop and forwarding table problems. These issues forced to do detailed troubleshooting and verification beyond the basic configuration checks that we knew. Despite these challenges, the lab ultimately achieved full route exchange and end-to-end connectivity. The troubleshooting process reinforced the distinction between route learning and packet forwarding, while highlighting the role of BGP attributes and redistribution behavior in environments with multiple protocols.

This lab improved my understanding of how BGP functions and strengthened my ability to diagnose complex routing issues.