Multiarea OSPF

Andrew Tang

Purpose

The concepts taught were configuring layer 3 switches and OSPF with multiple areas.

Background Information

OSPF, or Open Shortest Path First, is a routing protocol that, hence its name, uses the shortest path to send packets to their destination. It uses a neighbor system that maps out each devices neighbors and creates a map of the entire network. OSPF is reliable due to its fast response time and quick adaptation once a link goes down. It is also more efficient than some other routing methods because of this reason. It is widely used in medium and large networks, since it can choose the best possible route for packets without someone needing to configure it manually. In this lab, we used multiarea OSPF, which is a way of breaking a large OSPF network into smaller and more manageable parts, called areas. At the center of all the areas is the backbone area, or Area 0. All other areas must be directly or indirectly connected to it, as it acts like the main hub that connects the entire network. The reason for this need is because in a regular OSPF setup, every router has to track the entire map. This is fine for small networks but become inefficient in a large network. Routers will spend too much memory and processing power trying to keep up with all the details of the network. When the network is divided into areas, routers will only need to know the details of their own area. They still get information about the rest of the network, but it is summarized by special routers that connect areas together. Routers that sit on the borders of areas are called Area Border Routers (ABRs). They are the ones who translate and summarize information between detailed maps of areas and simplified summaries for other areas. This reduces the work strain put onto each router by reducing the amount of data that routers exchange. Thus, OSPF will run faster compared to a network using single-area OSPF. Additionally, networks using multiarea OSPF are easier to scale as they get bigger.

We also used layer 3 switches for part of this lab. A Layer 3 switch is a network device that combines the speed of a switch with the intelligence of a router. Normal switches (Layer 2) are mainly used to connect devices on the same local network, like computers in a computer lab. It uses MAC addresses to decide where to send data. A router (Layer 3), on the other hand, connects different networks together and makes decisions using IP addresses. Layer 3 switches can do both of these jobs. It can forward traffic very quickly in a local network and route traffic between different networks. They are extremely useful in large networks with many subnets that all need to communicate with each other efficiently. Instead of sending a lot of traffic to a separate router, which can create a bottleneck, a layer 3 switch can be used to handle routing directly and at much higher speeds.

Lab Summary

In this lab, we used 7 devices – 2 layer 3 switches and 5 routers. Our setup consisted of 2 layer 3 switches on one end of the network and the 5 routers mostly linearly aligned. We successfully configured multiarea OSPF, with 3 areas total and assigned both IPv4 and IPv6 addresses to each linked interface.

Lab Commands

The new commands that we learned for this lab are:

Router(config-if)# ipv6 ospf x area y

This command is used when configuring an interface to tell the router what OSPF protocol and area it is in, while also specifying that it should be using OSPF for IPv6 addresses.

Router(config-if)# ip ospf x area y

This command is used inside an interface configuration to tell the router the OSPF protocol number and the area that the interface is in. It also specifies that it should be using OSPF for IPv6 addresses.

Switch(config-if)# no switchport

This command was used on the Layer 3 switches, which told it that the ports are going to be Layer 3, which allows it to function as a router interface rather than a switch interface. This means that it will be able to route using layer 3 protocols.

(config-if)# ip routing

This command enables IPv4 routing on the device.

(config-if)# ipv6 unicast-routing

This command enables IPv6 routing on the device.

Network Diagram with IP’s

Configurations

Router 1 show run

version 16.9

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

platform punt-keepalive disable-kernel-core

hostname R1

boot-start-marker

boot-end-marker

vrf definition Mgmt-intf

address-family ipv4

exit-address-family

address-family ipv6

exit-address-family

no aaa new-model

login on-success log

subscriber templating

ipv6 unicast-routing

multilink bundle-name authenticated

crypto pki trustpoint TP-self-signed-1312775118

enrollment selfsigned

subject-name cn=IOS-Self-Signed-Certificate-1312775118

revocation-check none

rsakeypair TP-self-signed-1312775118

crypto pki certificate chain TP-self-signed-1312775118

certificate self-signed 01

30820330 30820218 A0030201 02020101 300D0609 2A864886 F70D0101 05050030

31312F30 2D060355 04031326 494F532D 53656C66 2D536967 6E65642D 43657274

69666963 6174652D 31333132 37373531 3138301E 170D3235 30393135 32313333

34335A17 0D333030 31303130 30303030 305A3031 312F302D 06035504 03132649

4F532D53 656C662D 5369676E 65642D43 65727469 66696361 74652D31 33313237

37353131 38308201 22300D06 092A8648 86F70D01 01010500 0382010F 00308201

A028201 01009129 E96360F6 988660C4 DB36A792 E1FAE4F3 B90E49AB E1E28C1E

96D52754 F4794100 7C1B4CE7 92B7259B 550D8DC7 D609FC15 4499AA23 BD8FF271

76BC50F5 B22EF457 0101669C 1309A522 71D6674C C5E32839 208D07EB F5B3E924

344A99E6 D3F00687 57D20D7C 571B12A7 A8E401A5 6DC6749D CC695108 10A4CE26

D2011FDF EA95FC02 4DF8A2DB 68DE79B1 E2EB9C28 1F3D105E 653D034F 68AD35B7

A910B5D3 E37D72D5 D0B2C0CB 0A183CA7 A4A656E3 2F8FDC56 DD8DBCC5 A1907CE1

1A568117 FB762921 E285B1C3 C2C1F853 EDACF031 C425414E E4DB7B86 1FEE0C7B

39AE6667 F980D539 A33CD758 DF5D263C 17AC0538 81E96D1F 93D42280 F2197492

1A185599 2D350203 010001A3 53305130 0F060355 1D130101 FF040530 030101FF

301F0603 551D2304 18301680 1494803F 398E98DE B2E01830 E878928F A2461655

11301D06 03551D0E 04160414 94803F39 8E98DEB2 E01830E8 78928FA2 46165511

300D0609 2A864886 F70D0101 05050003 82010100 7F12D1EE 5ECCEC3E B70A9C12

A7B0E81B 0BE52932 E46F843F 061D731F 9084806D B33E443A CF743D37 24F10728

6A5AF15F 3B7BCDBC 42C50B1C 2E8794C3 3CBBC21B F0BC2C5F 53EC3F3D 401120D4

F64446AB 86956957 9BCC4C55 C177CAB6 EE3C6E2B 58E37550 EC7DB500 EE566115

C8AC87D5 C993419E 1E1D735F CB96E1BE 7E3DEF90 DA998BDC 10EA41C1 FAFE858B

F6772FE8 2923C264 9F620476 13F3D2EF 122CA62B 86AFF876 2DC94F03 E0FEAB98

1B281ECF 36823424 54BADF0A 0295B494 3F1DE7EB 8C63EB8F 3B9BF216 B30A50DC

DF06E6D5 C8584951 D6E6142D 20C462E8 25CA2BC7 D774035A 39A4585D ABFA0C12

4EC303CB 2AF0DF45 B5B898A7 2F0B7F29 BAF130F3

quit

license udi pid ISR4321/K9 sn FDO21060Z52

no license smart enable

diagnostic bootup level minimal

spanning-tree extend system-id

redundancy

mode none

interface GigabitEthernet0/0/0

ip address 10.0.4.1 255.255.255.252

ip ospf 1 area 0

negotiation auto

ipv6 address 1004::1/64

ipv6 ospf 1 area 0

interface GigabitEthernet0/0/1

ip address 10.0.3.1 255.255.255.252

ip ospf 1 area 0

negotiation auto

ipv6 address 1003::1/64

ipv6 ospf 1 area 0

interface Serial0/1/0

interface Serial0/1/1

interface Service-Engine0/2/0

no ip address

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip address

shutdown

negotiation auto

router ospf 1

network 10.0.3.0 0.0.0.3 area 0

network 10.0.4.0 0.0.0.3 area 0

ip forward-protocol nd

ip http server

ip http authentication local

ip http secure-server

ip tftp source-interface GigabitEthernet0

ipv6 router ospf 1

control-plane

line con 0

transport input none

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

end

Router 1 routes

R1#show ip route

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2

ia - IS-IS inter area, * - candidate default, U - per-user static route

o - ODR, P - periodic downloaded static route, H - NHRP, l - LISP

a - application route

+ - replicated route, % - next hop override, p - overrides from PfR

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 8 subnets, 2 masks

O        10.0.2.0/30 [110/2] via 10.0.3.2, 00:33:26, GigabitEthernet0/0/1

C        10.0.3.0/30 is directly connected, GigabitEthernet0/0/1

L        10.0.3.1/32 is directly connected, GigabitEthernet0/0/1

C        10.0.4.0/30 is directly connected, GigabitEthernet0/0/0

L        10.0.4.1/32 is directly connected, GigabitEthernet0/0/0

O IA     10.1.1.0/30 [110/2] via 10.0.4.2, 00:49:10, GigabitEthernet0/0/0

O IA     10.1.2.0/30 [110/3] via 10.0.4.2, 00:48:25, GigabitEthernet0/0/0

O IA     10.9.1.0/30 [110/2] via 10.0.3.2, 00:33:27, GigabitEthernet0/0/1

R1#show ipv6 route

IPv6 Routing Table - default - 9 entries

Codes: C - Connected, L - Local, S - Static, U - Per-user Static route

B - BGP, R - RIP, I1 - ISIS L1, I2 - ISIS L2

IA - ISIS interarea, IS - ISIS summary, D - EIGRP, EX - EIGRP external

ND - ND Default, NDp - ND Prefix, DCE - Destination, NDr - Redirect

O - OSPF Intra, OI - OSPF Inter, OE1 - OSPF ext 1, OE2 - OSPF ext 2

ON1 - OSPF NSSA ext 1, ON2 - OSPF NSSA ext 2, a - Application

O   1002::/64 [110/2]

via FE80::C64D:84FF:FE2B:E5E4, GigabitEthernet0/0/1

C   1003::/64 [0/0]

via GigabitEthernet0/0/1, directly connected

L   1003::1/128 [0/0]

via GigabitEthernet0/0/1, receive

C   1004::/64 [0/0]

via GigabitEthernet0/0/0, directly connected

L   1004::1/128 [0/0]

via GigabitEthernet0/0/0, receive

OI  1011::/64 [110/2]

via FE80::267E:12FF:FE4D:E7B0, GigabitEthernet0/0/0

OI  1012::/64 [110/3]

via FE80::267E:12FF:FE4D:E7B0, GigabitEthernet0/0/0

OI  1091::/64 [110/2]

via FE80::C64D:84FF:FE2B:E5E4, GigabitEthernet0/0/1

L   FF00::/8 [0/0]

via Null0, receive

Router 2 show run

Building configuration...

Current configuration : 3718 bytes

!

! Last configuration change at 22:19:36 UTC Mon Sep 15 2025

!

version 16.9

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

platform punt-keepalive disable-kernel-core

!

hostname R2

!

boot-start-marker

boot-end-marker

!

!

vrf definition Mgmt-intf

!

address-family ipv4

exit-address-family

!

address-family ipv6

--More--

*Sep 15 22:19:36.821: %SYS-5-CONFIG_I: Configured from console  exit-address-family

!

!

no aaa new-model

!

!

!

!

login on-success log

!

!

!

!

!

!

!

subscriber templating

ipv6 unicast-routing

multilink bundle-name authenticated

!

!

!

crypto pki trustpoint TP-self-signed-3485044676

enrollment selfsigned

subject-name cn=IOS-Self-Signed-Certificate-3485044676

revocation-check none

rsakeypair TP-self-signed-3485044676

!

!

crypto pki certificate chain TP-self-signed-3485044676

certificate self-signed 01

30820330 30820218 A0030201 02020101 300D0609 2A864886 F70D0101 05050030

31312F30 2D060355 04031326 494F532D 53656C66 2D536967 6E65642D 43657274

69666963 6174652D 33343835 30343436 3736301E 170D3235 30393135 32313139

34375A17 0D333030 31303130 30303030 305A3031 312F302D 06035504 03132649

4F532D53 656C662D 5369676E 65642D43 65727469 66696361 74652D33 34383530

34343637 36308201 22300D06 092A8648 86F70D01 01010500 0382010F 00308201

0A028201 0100AF91 E8B99692 C090C5FE 305D1FB2 D3A3BA29 A376DE9E C3851717

49314BD6 0802D215 E322D8FF 6F800951 E27F0D88 018D854C DD9F43D1 779846FB

D5D27874 32EA70E4 B1CE9709 7220C170 81E5221D 743ED8D7 E375DAA8 B8C7A6C8

F15413C7 D2E62737 21E951BF 129CC8AC E3B9BADF 58DAF704 48D6FD86 BE7C979F

BDB48D9D D7B0E19D 88BD13C3 18F2DA18 3580023B E890FF27 5353DC73 FCF50906

9A0311A3 1EFE6015 C10DC42C E93BEA1E DDB9E9B2 B47DD215 2BCBFCBE 2C9BE3A0

11FB74F4 C2B187C5 31DAA135 45D75F51 DE600C31 5C103F3C 7DF03E53 ED033409

25ACE23F 2613AD9C E4A968E4 2680DF6A 62A4C681 D6771340 FBCBC901 2CD3AB70

8DFBC4D8 DCB10203 010001A3 53305130 0F060355 1D130101 FF040530 030101FF

301F0603 551D2304 18301680 14499C1D 4B87583E 7B1BF580 DDF75403 D4117D1C

95301D06 03551D0E 04160414 499C1D4B 87583E7B 1BF580DD F75403D4 117D1C95

300D0609 2A864886 F70D0101 05050003 82010100 A4FD85A7 3E5080FE CEFD37D7

54ECCA89 835F3646 37B72D0C 0F41EBC2 865E837C 49E6DF79 62AB9E49 7BCCFA1F

EC6FB44C CA744F5E 2FFFCB17 B0D6E77D 9506FE7C 67322048 EEB8FC90 15E6C1E8

E5260830 17FF22D5 8B44774A 09C20F4E 961B5FCB B830DC74 07D2239A EB2C42B8

C8D11B96 037C76C0 7352D035 D20CFCF9 91F2A030 0E1364A6 2642791D 591D6FE8

4EF8141C 8F266ABB 4B196C4B D96CAC52 C74A6184 CE5EB6D6 D40FDA69 134C9E46

3B8EDBC3 BEB41CF6 C97328D5 3C61220D DE04F1F3 BE546CEC 497552A7 F75F9B6A

FD285077 9B2C5C15 5DFAD4C8 B6E09114 E6D2EB02 5009E751 01FABFBF 9EA6E270

B4706B46 B024839E 48D4FBA0 5F8DBFC9 271141A4

quit

!

license udi pid ISR4321/K9 sn FDO21082HVD

no license smart enable

diagnostic bootup level minimal

!

spanning-tree extend system-id

!

!

!

!

redundancy

mode none

!

!

!

!

!

!

!

!

interface GigabitEthernet0/0/0

no ip address

negotiation auto

!

interface GigabitEthernet0/0/1

ip address 10.0.2.1 255.255.255.252

negotiation auto

ipv6 address 1002::1/64

ipv6 ospf 1 area 0

!

interface Serial0/1/0

no ip address

shutdown

!

interface Serial0/1/1

no ip address

shutdown

!

interface Service-Engine0/2/0

no ip address

!

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip address

shutdown

negotiation auto

!

router ospf 1

network 10.0.2.0 0.0.0.3 area 0

!

ip forward-protocol nd

ip http server

ip http authentication local

ip http secure-server

ip tftp source-interface GigabitEthernet0

!

!

ipv6 router ospf 1

!

!

!

!

!

control-plane

!

!

line con 0

transport input none

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

!

!

!

!

!

!

End

Router 2 routes

R2#show ip route

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2

ia - IS-IS inter area, * - candidate default, U - per-user static route

o - ODR, P - periodic downloaded static route, H - NHRP, l - LISP

a - application route

+ - replicated route, % - next hop override, p - overrides from PfR

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 7 subnets, 2 masks

C        10.0.2.0/30 is directly connected, GigabitEthernet0/0/1

L        10.0.2.1/32 is directly connected, GigabitEthernet0/0/1

O        10.0.3.0/30 [110/2] via 10.0.2.2, 00:34:02, GigabitEthernet0/0/1

O        10.0.4.0/30 [110/3] via 10.0.2.2, 00:34:02, GigabitEthernet0/0/1

O IA     10.1.1.0/30 [110/4] via 10.0.2.2, 00:34:02, GigabitEthernet0/0/1

O IA     10.1.2.0/30 [110/5] via 10.0.2.2, 00:34:02, GigabitEthernet0/0/1

O IA     10.9.1.0/30 [110/2] via 10.0.2.2, 00:34:02, GigabitEthernet0/0/1

R2#show ipv

R2#show ipv6 route

IPv6 Routing Table - default - 8 entries

Codes: C - Connected, L - Local, S - Static, U - Per-user Static route

B - BGP, R - RIP, I1 - ISIS L1, I2 - ISIS L2

IA - ISIS interarea, IS - ISIS summary, D - EIGRP, EX - EIGRP external

ND - ND Default, NDp - ND Prefix, DCE - Destination, NDr - Redirect

O - OSPF Intra, OI - OSPF Inter, OE1 - OSPF ext 1, OE2 - OSPF ext 2

ON1 - OSPF NSSA ext 1, ON2 - OSPF NSSA ext 2, a - Application

C   1002::/64 [0/0]

via GigabitEthernet0/0/1, directly connected

L   1002::1/128 [0/0]

via GigabitEthernet0/0/1, receive

O   1003::/64 [110/2]

via FE80::C64D:84FF:FE2B:E5D6, GigabitEthernet0/0/1

O   1004::/64 [110/3]

via FE80::C64D:84FF:FE2B:E5D6, GigabitEthernet0/0/1

OI  1011::/64 [110/4]

via FE80::C64D:84FF:FE2B:E5D6, GigabitEthernet0/0/1

OI  1012::/64 [110/5]

via FE80::C64D:84FF:FE2B:E5D6, GigabitEthernet0/0/1

OI  1091::/64 [110/2]

via FE80::C64D:84FF:FE2B:E5D6, GigabitEthernet0/0/1

L   FF00::/8 [0/0]

via Null0, receive

Router 3 show run

Building configuration...

Current configuration : 3730 bytes

!

! Last configuration change at 21:51:35 UTC Mon Sep 15 2025

!

version 16.9

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

platform punt-keepalive disable-kernel-core

!

hostname R3

!

boot-start-marker

boot-end-marker

!

!

vrf definition Mgmt-intf

!

address-family ipv4

exit-address-family

!

address-family ipv6

exit-address-family

!

!

no aaa new-model

!

!

!

!

login on-success log

!

!

!

!

!

!

!

subscriber templating

ipv6 unicast-routing

multilink bundle-name authenticated

!

!

!

crypto pki trustpoint TP-self-signed-2857297660

enrollment selfsigned

subject-name cn=IOS-Self-Signed-Certificate-2857297660

revocation-check none

rsakeypair TP-self-signed-2857297660

!

!

crypto pki certificate chain TP-self-signed-2857297660

certificate self-signed 01

30820330 30820218 A0030201 02020101 300D0609 2A864886 F70D0101 05050030

31312F30 2D060355 04031326 494F532D 53656C66 2D536967 6E65642D 43657274

69666963 6174652D 32383537 32393736 3630301E 170D3235 30393135 32313434

34355A17 0D333030 31303130 30303030 305A3031 312F302D 06035504 03132649

4F532D53 656C662D 5369676E 65642D43 65727469 66696361 74652D32 38353732

39373636 30308201 22300D06 092A8648 86F70D01 01010500 0382010F 00308201

0A028201 0100DD5D 01C0E332 9EE196F3 51502466 5E262C59 58A5311E 7FEFF3E2

0B90D5D3 BD8D7AB7 BA4269C6 153E9A18 8FE69FC6 D3BB3937 50E37B28 8D5F5E51

6237B9EA 8BCB0A5F 75F63092 4F71878F CC19C5DC 0E96EE9E DC8DD19F 4651E86E

318123E4 81C63FB4 F447F2A5 8EDCA952 42CEF3E5 C604901F 26CABE47 E19ACEF9

DE9FCD6C A983E9EB C6E2DD94 71F19B03 F76AA356 16AD8CDA 6E7E9644 FF95A70C

C183544F 95254EA5 2152FEE6 1778D06F 73A1AA31 09FBE76F 367C0038 41FCC0A7

22F14378 C29D4D54 6F7FC45D 96C7D902 36E225BC 1809F7A0 47C4583D 24E53D4D

4F4C03AB 05C1049A 58C65057 6628F38D 890593D3 899984F7 E07F298D 9884F959

D9405134 5A6B0203 010001A3 53305130 0F060355 1D130101 FF040530 030101FF

301F0603 551D2304 18301680 14E95ADD 5F616CA9 2552A94D 7F2270B2 E7EAA3B4

B5301D06 03551D0E 04160414 E95ADD5F 616CA925 52A94D7F 2270B2E7 EAA3B4B5

300D0609 2A864886 F70D0101 05050003 82010100 D897679C F5856AAF 9C5814D0

121C5FD5 D003EB03 B264062A 89A3CF4E 584B2525 68D8DB30 31803FF5 E357C727

B0272316 7734FEA0 3125BE91 0AAF44C3 AF72C9E9 5D6D60BB A2177DA8 34620828

E14B5514 DFC6ECC9 D10E87AA 421A2DF4 8DB2F57B B625C6C3 66DB7770 CC3C5102

F3FB4078 5620D741 BBECDCD9 D387FBCE 8F9DB2F3 92AACB4D 536DCA0D 1ACDDACB

E83D86E0 C6355673 7BA33C52 F8BA0AAC B6062C18 EF9BA33C B865D309 199D28CE

4A6204A1 EBFAF75E 0EE45A06 EA3FCB1B 093B9322 CC821A9B CB0F3AAE 8B016B72

2BC1B82F 956D3EE2 AE432DD9 7FFF3420 DC89979F F72D9D4D 9D2B866B EAB6641A

B445D369 AEF53766 F9CE5E5F D3D04050 C8CC6C01

quit

!

license udi pid ISR4321/K9 sn FDO21500FNQ

no license smart enable

diagnostic bootup level minimal

!

spanning-tree extend system-id

!

!

!

!

redundancy

mode none

!

!

!

!

!

!

!

!

interface GigabitEthernet0/0/0

ip address 10.1.1.2 255.255.255.252

negotiation auto

ipv6 address 1011::2/64

ipv6 ospf 1 area 1

!

interface GigabitEthernet0/0/1

ip address 10.1.2.2 255.255.255.252

negotiation auto

ipv6 address 1012::2/64

ipv6 ospf 1 area 1

!

interface Service-Engine0/2/0

no ip address

shutdown

!

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip address

shutdown

negotiation auto

!

router ospf 1

network 10.1.1.0 0.0.0.3 area 1

network 10.1.2.0 0.0.0.3 area 1

!

ip forward-protocol nd

ip http server

ip http authentication local

ip http secure-server

ip tftp source-interface GigabitEthernet0

!

!

ipv6 router ospf 1

!

!

!

!

!

control-plane

!

!

line con 0

transport input none

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

!

!

!

!

!

!

End

Router 3 routes

R3#show ip route

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2

ia - IS-IS inter area, * - candidate default, U - per-user static route

o - ODR, P - periodic downloaded static route, H - NHRP, l - LISP

a - application route

+ - replicated route, % - next hop override, p - overrides from PfR

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 8 subnets, 2 masks

O IA     10.0.2.0/30 [110/4] via 10.1.1.1, 00:34:30, GigabitEthernet0/0/0

O IA     10.0.3.0/30 [110/3] via 10.1.1.1, 00:40:05, GigabitEthernet0/0/0

O IA     10.0.4.0/30 [110/2] via 10.1.1.1, 00:49:30, GigabitEthernet0/0/0

C        10.1.1.0/30 is directly connected, GigabitEthernet0/0/0

L        10.1.1.2/32 is directly connected, GigabitEthernet0/0/0

C        10.1.2.0/30 is directly connected, GigabitEthernet0/0/1

L        10.1.2.2/32 is directly connected, GigabitEthernet0/0/1

O IA     10.9.1.0/30 [110/4] via 10.1.1.1, 00:34:30, GigabitEthernet0/0/0

R3#show ipv6 route

IPv6 Routing Table - default - 9 entries

Codes: C - Connected, L - Local, S - Static, U - Per-user Static route

B - BGP, R - RIP, I1 - ISIS L1, I2 - ISIS L2

IA - ISIS interarea, IS - ISIS summary, D - EIGRP, EX - EIGRP external

ND - ND Default, NDp - ND Prefix, DCE - Destination, NDr - Redirect

O - OSPF Intra, OI - OSPF Inter, OE1 - OSPF ext 1, OE2 - OSPF ext 2

ON1 - OSPF NSSA ext 1, ON2 - OSPF NSSA ext 2, a - Application

OI  1002::/64 [110/4]

via FE80::267E:12FF:FE4D:E7B1, GigabitEthernet0/0/0

OI  1003::/64 [110/3]

via FE80::267E:12FF:FE4D:E7B1, GigabitEthernet0/0/0

OI  1004::/64 [110/2]

via FE80::267E:12FF:FE4D:E7B1, GigabitEthernet0/0/0

C   1011::/64 [0/0]

via GigabitEthernet0/0/0, directly connected

L   1011::2/128 [0/0]

via GigabitEthernet0/0/0, receive

C   1012::/64 [0/0]

via GigabitEthernet0/0/1, directly connected

L   1012::2/128 [0/0]

via GigabitEthernet0/0/1, receive

OI  1091::/64 [110/4]

via FE80::267E:12FF:FE4D:E7B1, GigabitEthernet0/0/0

L   FF00::/8 [0/0]

via Null0, receive

Router 4 show run

Building configuration...

Current configuration : 3662 bytes

!

! Last configuration change at 21:59:15 UTC Mon Sep 15 2025

!

version 16.9

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

platform punt-keepalive disable-kernel-core

!

hostname R4

!

boot-start-marker

boot-end-marker

!

!

vrf definition Mgmt-intf

!

address-family ipv4

exit-address-family

!

address-family ipv6

exit-address-family

!

!

no aaa new-model

!

!

!

!

login on-success log

!

!

!

!

!

!

!

subscriber templating

ipv6 unicast-routing

multilink bundle-name authenticated

!

!

!

crypto pki trustpoint TP-self-signed-1192009516

enrollment selfsigned

subject-name cn=IOS-Self-Signed-Certificate-1192009516

revocation-check none

rsakeypair TP-self-signed-1192009516

!

!

crypto pki certificate chain TP-self-signed-1192009516

certificate self-signed 01

30820330 30820218 A0030201 02020101 300D0609 2A864886 F70D0101 05050030

31312F30 2D060355 04031326 494F532D 53656C66 2D536967 6E65642D 43657274

69666963 6174652D 31313932 30303935 3136301E 170D3235 30393135 32313234

32365A17 0D333030 31303130 30303030 305A3031 312F302D 06035504 03132649

4F532D53 656C662D 5369676E 65642D43 65727469 66696361 74652D31 31393230

30393531 36308201 22300D06 092A8648 86F70D01 01010500 0382010F 00308201

0A028201 0100A920 52C6CD73 E7A7F1F3 18C3ECEC 9814BB04 B8BC5A32 99AB5C4B

F18246BE CA2ED3E5 82C74AC0 0069BBD9 29F30F37 893D83EF 10751E57 9CDEED47

4A51DC63 D58234B9 47FDC8A3 C2367F00 28D61418 839B5BE7 806C38FA 035B72BC

A8AFE3B4 B840DFA8 EF1B2749 5538B954 1A858A3A EF5EAEC7 171367A1 F5F357C7

E08DCB43 83D6233A 710CA8F4 F70F9643 8C097A9E 72FDC5E3 86060700 66CBDCF3

F5CBB06A 32AEBD41 F696FD22 01A0B7EA FD55423B BBB29B08 97EE4020 3E7F0AA4

75AB413D 0434746B 7FB06C77 E545ACC9 284B4175 EA4DEBF7 92606BAA BD5B74BA

AA591472 1E84B3DE 27F738BE 35819203 BBD2E56F BBB53190 7C3AE134 096A253D

1F26CD35 23F10203 010001A3 53305130 0F060355 1D130101 FF040530 030101FF

301F0603 551D2304 18301680 146B9E84 37ACDA49 F1BF9583 9A7DFB34 3BB71669

30301D06 03551D0E 04160414 6B9E8437 ACDA49F1 BF95839A 7DFB343B B7166930

300D0609 2A864886 F70D0101 05050003 82010100 8A25F866 EDA87B09 22622186

52FE527B 1D02BD4F 60854B66 5823C869 40DF1EE9 2ACEC3E6 03B6137C 3D087CF5

B9EFC451 86448D75 79FB60BB 32A882E5 52DFF72B B205559F C4ADAC8F ED21CDDD

747ADDC1 05587B34 5E75941C B5F93A89 065B0964 E3B414E2 3AB56A4E AF200343

7D5F7B59 7FB410CB 1BFA5174 85B8C1E7 9DBA372A 64076473 732ACE71 6DEF9922

EAEF99C2 895C9077 668D6CD0 8B61E763 56D89405 A7A223C9 1F4A9BF9 F46D78B9

515B226C 30D8D4CF CAF9FC55 B8970729 D9D4F9A8 883D3AFF 0724F0AA 6136DCF1

A9A3F188 0639844C EB740F8F EAECB0DF B939C859 4F9364C6 872722D7 86938F6F

DC23328A DDE29BB9 C5F7C543 A1B7BCA2 8F515A8D

quit

!

license udi pid ISR4321/K9 sn FDO21482DVS

no license smart enable

diagnostic bootup level minimal

!

spanning-tree extend system-id

!

!

!

!

redundancy

mode none

!

!

!

!

!

!

!

!

interface GigabitEthernet0/0/0

ip address 10.0.4.2 255.255.255.252

ip ospf 1 area 0

negotiation auto

ipv6 address 1004::2/64

ipv6 ospf 1 area 0

!

interface GigabitEthernet0/0/1

ip address 10.1.1.1 255.255.255.252

ip ospf 1 area 1

negotiation auto

ipv6 address 1011::1/64

ipv6 ospf 1 area 1

!

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip address

shutdown

negotiation auto

!

router ospf 1

router-id 4.4.4.4

!

ip forward-protocol nd

ip http server

ip http authentication local

ip http secure-server

ip tftp source-interface GigabitEthernet0

!

!

ipv6 router ospf 1

!

!

!

!

!

control-plane

!

!

line con 0

transport input none

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

!

!

!

!

!

!

End

Router 4 routes

R4#show ip route

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2

ia - IS-IS inter area, * - candidate default, U - per-user static route

o - ODR, P - periodic downloaded static route, H - NHRP, l - LISP

a - application route

+ - replicated route, % - next hop override, p - overrides from PfR

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 8 subnets, 2 masks

O        10.0.2.0/30 [110/3] via 10.0.4.1, 00:34:59, GigabitEthernet0/0/0

O        10.0.3.0/30 [110/2] via 10.0.4.1, 00:40:43, GigabitEthernet0/0/0

C        10.0.4.0/30 is directly connected, GigabitEthernet0/0/0

L        10.0.4.2/32 is directly connected, GigabitEthernet0/0/0

C        10.1.1.0/30 is directly connected, GigabitEthernet0/0/1

L        10.1.1.1/32 is directly connected, GigabitEthernet0/0/1

O        10.1.2.0/30 [110/2] via 10.1.1.2, 00:49:59, GigabitEthernet0/0/1

O IA     10.9.1.0/30 [110/3] via 10.0.4.1, 00:34:59, GigabitEthernet0/0/0

R4#show ipv6 route

IPv6 Routing Table - default - 9 entries

Codes: C - Connected, L - Local, S - Static, U - Per-user Static route

B - BGP, R - RIP, I1 - ISIS L1, I2 - ISIS L2

IA - ISIS interarea, IS - ISIS summary, D - EIGRP, EX - EIGRP external

ND - ND Default, NDp - ND Prefix, DCE - Destination, NDr - Redirect

O - OSPF Intra, OI - OSPF Inter, OE1 - OSPF ext 1, OE2 - OSPF ext 2

ON1 - OSPF NSSA ext 1, ON2 - OSPF NSSA ext 2, a - Application

O   1002::/64 [110/3]

via FE80::72D3:79FF:FE02:8160, GigabitEthernet0/0/0

O   1003::/64 [110/2]

via FE80::72D3:79FF:FE02:8160, GigabitEthernet0/0/0

C   1004::/64 [0/0]

via GigabitEthernet0/0/0, directly connected

L   1004::2/128 [0/0]

via GigabitEthernet0/0/0, receive

C   1011::/64 [0/0]

via GigabitEthernet0/0/1, directly connected

L   1011::1/128 [0/0]

via GigabitEthernet0/0/1, receive

O   1012::/64 [110/2]

via FE80::521C:B0FF:FE63:37A0, GigabitEthernet0/0/1

OI  1091::/64 [110/3]

via FE80::72D3:79FF:FE02:8160, GigabitEthernet0/0/0

L   FF00::/8 [0/0]

via Null0, receive

Router 5 show run

Building configuration...

Current configuration : 3720 bytes

!

! Last configuration change at 21:34:56 UTC Mon Sep 15 2025

!

version 16.9

service timestamps debug datetime msec

service timestamps log datetime msec

platform qfp utilization monitor load 80

platform punt-keepalive disable-kernel-core

!

hostname R5

!

boot-start-marker

boot-end-marker

!

!

vrf definition Mgmt-intf

!

address-family ipv4

exit-address-family

!

address-family ipv6

exit-address-family

!

!

no aaa new-model

!

!

!

!

login on-success log

!

!

!

!

!

!

!

subscriber templating

ipv6 unicast-routing

multilink bundle-name authenticated

!

!

!

crypto pki trustpoint TP-self-signed-3875574019

enrollment selfsigned

subject-name cn=IOS-Self-Signed-Certificate-3875574019

revocation-check none

rsakeypair TP-self-signed-3875574019

!

!

crypto pki certificate chain TP-self-signed-3875574019

certificate self-signed 01

30820330 30820218 A0030201 02020101 300D0609 2A864886 F70D0101 05050030

31312F30 2D060355 04031326 494F532D 53656C66 2D536967 6E65642D 43657274

69666963 6174652D 33383735 35373430 3139301E 170D3235 30393135 32313333

34345A17 0D333030 31303130 30303030 305A3031 312F302D 06035504 03132649

4F532D53 656C662D 5369676E 65642D43 65727469 66696361 74652D33 38373535

37343031 39308201 22300D06 092A8648 86F70D01 01010500 0382010F 00308201

0A028201 0100C1F5 32DC40AB 083A810C 04EC62BC F1915B12 7C919B17 22716734

B56207FF CA41D301 C1772D56 005B1CE0 6FB6F0ED 70877A39 25B7B5CB 03B52858

DCA87091 8560546D 6796B2D2 EF40F192 AEF089FA DAB50562 4A32A568 41639919

9FF9FD25 75AB45DF 59805331 E249B436 7E2E3E15 B5382CE6 D2F37515 C3FB6FF4

FACF51BC 257C7A29 6CAB5ED9 86C74E4C 3F8ABBBF A068F4B4 7071FBF4 AB074E0A

35E97E7E E433FCE1 DA88FDD2 7FAB8AFB 31DBAC48 E828ED46 6BD0B7C9 6F5CFA21

C0231A20 E90C7DFA 09B68F76 2F8734BF 262A85CB 2A956E08 5411ACF5 32353E2D

0960C5C0 9D52538D DD6FF91D 84369AEE C65F993C 893DD925 869EB60C 6ECCCE74

A4CA1D17 C6A10203 010001A3 53305130 0F060355 1D130101 FF040530 030101FF

301F0603 551D2304 18301680 1488EC2F B337DCC2 5C20163B 68636F62 D4DE0EA6

B4301D06 03551D0E 04160414 88EC2FB3 37DCC25C 20163B68 636F62D4 DE0EA6B4

300D0609 2A864886 F70D0101 05050003 82010100 623BD748 E59E18F4 156317E3

9F46B22A 80343353 88348537 3400524B 4C733CF9 DC6ABD19 FC4EA066 CD065ABB

84CC772F 7F24FA7A 9C348819 EFE9B7B3 C854B35E 8DE6B959 1ACB4873 35F1A055

2BCD8EE8 DD3D0CDF 01058346 212F6EEC 08179784 50F3092C E2CA4482 5A2A6171

C9BD6ECE 58FBDE8E 85D15244 8E8C438D 60B8DC06 2B7BA9BB 76830AB9 97BFD352

C40980D0 CB843E78 5C1C01A3 57881987 9C2B2E55 3515D621 8BDDE945 0DED7252

8363F85B 36C0C3A0 220744F6 C017F26F 0ED9203F 3E92551D CF7A5A9C 30638713

64235B15 E0D7A984 BF38BC15 DC2AF0B1 B474E84D 211CBBE6 3DC40608 40C63407

E7619B8A FB378082 E463B20A 654956EC 05C776C2

quit

!

license udi pid ISR4321/K9 sn FLM2407014D

no license smart enable

diagnostic bootup level minimal

!

spanning-tree extend system-id

!

!

!

!

redundancy

mode none

!

!

!

!

!

!

!

!

interface GigabitEthernet0/0/0

ip address 10.1.2.1 255.255.255.252

ip ospf 1 area 1

negotiation auto

ipv6 address 1012::1/64

ipv6 ospf 1 area 1

!

interface GigabitEthernet0/0/1

no ip address

shutdown

negotiation auto

!

interface GigabitEthernet0/2/0

no ip address

shutdown

negotiation auto

!

interface GigabitEthernet0/2/1

no ip address

shutdown

negotiation auto

!

interface GigabitEthernet0

vrf forwarding Mgmt-intf

no ip address

shutdown

negotiation auto

!

router ospf 1

!

ip forward-protocol nd

ip http server

ip http authentication local

ip http secure-server

ip tftp source-interface GigabitEthernet0

!

!

ipv6 router ospf 1

!

!

!

!

!

control-plane

!

!

line con 0

transport input none

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

!

!

!

!

!

!

end

Router 5 routes

R5#show ip route

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2

ia - IS-IS inter area, * - candidate default, U - per-user static route

o - ODR, P - periodic downloaded static route, H - NHRP, l - LISP

a - application route

+ - replicated route, % - next hop override, p - overrides from PfR

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 7 subnets, 2 masks

O IA     10.0.2.0/30 [110/5] via 10.1.2.2, 00:35:26, GigabitEthernet0/0/0

O IA     10.0.3.0/30 [110/4] via 10.1.2.2, 00:41:00, GigabitEthernet0/0/0

O IA     10.0.4.0/30 [110/3] via 10.1.2.2, 00:50:22, GigabitEthernet0/0/0

O        10.1.1.0/30 [110/2] via 10.1.2.2, 00:51:10, GigabitEthernet0/0/0

C        10.1.2.0/30 is directly connected, GigabitEthernet0/0/0

L        10.1.2.1/32 is directly connected, GigabitEthernet0/0/0

O IA     10.9.1.0/30 [110/5] via 10.1.2.2, 00:35:26, GigabitEthernet0/0/0

R5#show ipv6 route

IPv6 Routing Table - default - 8 entries

Codes: C - Connected, L - Local, S - Static, U - Per-user Static route

B - BGP, R - RIP, I1 - ISIS L1, I2 - ISIS L2

IA - ISIS interarea, IS - ISIS summary, D - EIGRP, EX - EIGRP external

ND - ND Default, NDp - ND Prefix, DCE - Destination, NDr - Redirect

O - OSPF Intra, OI - OSPF Inter, OE1 - OSPF ext 1, OE2 - OSPF ext 2

ON1 - OSPF NSSA ext 1, ON2 - OSPF NSSA ext 2, a - Application

OI  1002::/64 [110/5]

via FE80::521C:B0FF:FE63:37A1, GigabitEthernet0/0/0

OI  1003::/64 [110/4]

via FE80::521C:B0FF:FE63:37A1, GigabitEthernet0/0/0

OI  1004::/64 [110/3]

via FE80::521C:B0FF:FE63:37A1, GigabitEthernet0/0/0

O   1011::/64 [110/2]

via FE80::521C:B0FF:FE63:37A1, GigabitEthernet0/0/0

C   1012::/64 [0/0]

via GigabitEthernet0/0/0, directly connected

L   1012::1/128 [0/0]

via GigabitEthernet0/0/0, receive

OI  1091::/64 [110/5]

via FE80::521C:B0FF:FE63:37A1, GigabitEthernet0/0/0

L   FF00::/8 [0/0]

via Null0, receive

S1 show run

Building configuration...

Current configuration : 8829 bytes

!

! Last configuration change at 21:53:52 UTC Mon Sep 15 2025

!

version 16.9

no service pad

service timestamps debug datetime msec

service timestamps log datetime msec

! Call-home is enabled by Smart-Licensing.

service call-home

no platform punt-keepalive disable-kernel-core

!

hostname S1

!

!

vrf definition Mgmt-vrf

!

address-family ipv4

exit-address-family

!

address-family ipv6

exit-address-family

!

!

!

!

!

no aaa new-model

switch 1 provision c9200l-24t-4g

!

!

!

!

!

!

!

!

!

ip routing

ip arp entry learn 10240

!

!

!

!

login on-success log

ipv6 unicast-routing

call-home

! If contact email address in call-home is configured as sch-smart-licensing@cisco.com

! the email address configured in Cisco Smart License Portal will be used as contact email address to send SCH notifications.

contact-email-addr sch-smart-licensing@cisco.com

profile "CiscoTAC-1"

active

destination transport-method http

no destination transport-method email

!

crypto pki trustpoint TP-self-signed-3161893787

enrollment selfsigned

subject-name cn=IOS-Self-Signed-Certificate-3161893787

revocation-check none

rsakeypair TP-self-signed-3161893787

!

crypto pki trustpoint SLA-TrustPoint

enrollment pkcs12

revocation-check crl

!

!

crypto pki certificate chain TP-self-signed-3161893787

certificate self-signed 01

30820330 30820218 A0030201 02020101 300D0609 2A864886 F70D0101 05050030

31312F30 2D060355 04031326 494F532D 53656C66 2D536967 6E65642D 43657274

69666963 6174652D 33313631 38393337 3837301E 170D3235 30393135 32313239

32365A17 0D333030 31303130 30303030 305A3031 312F302D 06035504 03132649

4F532D53 656C662D 5369676E 65642D43 65727469 66696361 74652D33 31363138

39333738 37308201 22300D06 092A8648 86F70D01 01010500 0382010F 00308201

0A028201 0100D743 86D80366 572A1801 EC4A23BC 7DF1AE08 711D3493 2A0E3898

FB368E77 28FBCA03 0BD916E3 B4E19356 5BAD3879 147E31D1 F66A709B 26DCE6A2

944A3346 FC656904 085647F2 28BC40CE A418967E AB7264DC BDADC7C4 92308CF2

9EDB4D75 07D900E6 80B6184F 0706676A 7C2B27FE D810943B 35A84F53 F5ACE829

D4894717 2472C3FC 44119B3A 174525E7 3D81339C C98D7BFC 5EF735CE 94BC4B0D

A860AFAC 88A51D53 5ABBC585 4DC26A7E 1DB614D8 473AF621 650E2F14 5DFEA01E

1810ACBC 168C2746 72104FFA DDAC4891 FE21C688 ADB725F2 8D1ABCF7 D735EEE2

83CD1FF7 D0EB5EA0 72913BE1 8E636809 401DEECD F165621B 9B676034 6258515A

702E5AEF 8CD30203 010001A3 53305130 0F060355 1D130101 FF040530 030101FF

301F0603 551D2304 18301680 14B246FE F808D8BF 72193393 5ED22D34 35DCDACB

CB301D06 03551D0E 04160414 B246FEF8 08D8BF72 1933935E D22D3435 DCDACBCB

300D0609 2A864886 F70D0101 05050003 82010100 68442370 8BFC63AA C66E3CC3

3705680A 4A833E6F 98885D85 7FE41276 858D77A0 72076535 BE7B6B86 1F30E494

4B35A48D 75D11B29 D00FF03A C4A934D7 55BE301F 3FD3719F 2FB26D8F C1535429

D65330E6 E517672A 2379B631 5EA50C6E E0A90169 95C2F2A4 2AD218E3 BAC242F9

6CFAE9D2 47319EBD 9439D12A 5B5E54B0 21A6CFF1 C73EFAC9 D4E25E29 82546B33

39999D2B 9DABF8E5 D275931D 87B3B784 4206985F E155CC0C DD2C9B3B 5B0B2743

12D192A5 3BB44782 EE10B678 FEA6C7D0 ED8B4314 1F932E30 FD2E17B0 98FE422C

1138FF1C B8988EAB 99D37414 C0D8766A A4EF1F2D C928377B 0399CF8A D68BD482

ED350795 D02221BD AD746320 F0D98E30 7BD60DCD

quit

crypto pki certificate chain SLA-TrustPoint

certificate ca 01

30820321 30820209 A0030201 02020101 300D0609 2A864886 F70D0101 0B050030

32310E30 0C060355 040A1305 43697363 6F312030 1E060355 04031317 43697363

6F204C69 63656E73 696E6720 526F6F74 20434130 1E170D31 33303533 30313934

3834375A 170D3338 30353330 31393438 34375A30 32310E30 0C060355 040A1305

43697363 6F312030 1E060355 04031317 43697363 6F204C69 63656E73 696E6720

526F6F74 20434130 82012230 0D06092A 864886F7 0D010101 05000382 010F0030

82010A02 82010100 A6BCBD96 131E05F7 145EA72C 2CD686E6 17222EA1 F1EFF64D

CBB4C798 212AA147 C655D8D7 9471380D 8711441E 1AAF071A 9CAE6388 8A38E520

1C394D78 462EF239 C659F715 B98C0A59 5BBB5CBD 0CFEBEA3 700A8BF7 D8F256EE

4AA4E80D DB6FD1C9 60B1FD18 FFC69C96 6FA68957 A2617DE7 104FDC5F EA2956AC

7390A3EB 2B5436AD C847A2C5 DAB553EB 69A9A535 58E9F3E3 C0BD23CF 58BD7188

68E69491 20F320E7 948E71D7 AE3BCC84 F10684C7 4BC8E00F 539BA42B 42C68BB7

C7479096 B4CB2D62 EA2F505D C7B062A4 6811D95B E8250FC4 5D5D5FB8 8F27D191

C55F0D76 61F9A4CD 3D992327 A8BB03BD 4E6D7069 7CBADF8B DF5F4368 95135E44

DFC7C6CF 04DD7FD1 02030100 01A34230 40300E06 03551D0F 0101FF04 04030201

06300F06 03551D13 0101FF04 05300301 01FF301D 0603551D 0E041604 1449DC85

4B3D31E5 1B3E6A17 606AF333 3D3B4C73 E8300D06 092A8648 86F70D01 010B0500

03820101 00507F24 D3932A66 86025D9F E838AE5C 6D4DF6B0 49631C78 240DA905

604EDCDE FF4FED2B 77FC460E CD636FDB DD44681E 3A5673AB 9093D3B1 6C9E3D8B

D98987BF E40CBD9E 1AECA0C2 2189BB5C 8FA85686 CD98B646 5575B146 8DFC66A8

467A3DF4 4D565700 6ADF0F0D CF835015 3C04FF7C 21E878AC 11BA9CD2 55A9232C

7CA7B7E6 C1AF74F6 152E99B7 B1FCF9BB E973DE7F 5BDDEB86 C71E3B49 1765308B

5FB0DA06 B92AFE7F 494E8A9E 07B85737 F3A58BE1 1A48A229 C37C1E69 39F08678

80DDCD16 D6BACECA EEBC7CF9 8428787B 35202CDC 60E4616A B623CDBD 230E3AFB

418616A9 4093E049 4D10AB75 27E86F73 932E35B5 8862FDAE 0275156F 719BB2F0

D697DF7F 28

quit

!

license boot level network-essentials addon dna-essentials

!

!

diagnostic bootup level minimal

!

spanning-tree mode rapid-pvst

spanning-tree extend system-id

!

!

!

redundancy

mode sso

!

!

!

class-map match-any system-cpp-police-topology-control

description Topology control

class-map match-any system-cpp-police-sw-forward

description Sw forwarding, L2 LVX data, LOGGING

class-map match-any system-cpp-default

description Inter FED, EWLC control, EWLC data

class-map match-any system-cpp-police-sys-data

description Learning cache ovfl, High Rate App, Exception, EGR Exception, NFL SAMPLED DATA, RPF Failed

class-map match-any system-cpp-police-punt-webauth

description Punt Webauth

class-map match-any system-cpp-police-l2lvx-control

description L2 LVX control packets

class-map match-any system-cpp-police-forus

description Forus Address resolution and Forus traffic

class-map match-any system-cpp-police-multicast-end-station

description MCAST END STATION

class-map match-any system-cpp-police-multicast

description Transit Traffic and MCAST Data

class-map match-any system-cpp-police-l2-control

description L2 control

class-map match-any system-cpp-police-dot1x-auth

description DOT1X Auth

class-map match-any system-cpp-police-data

description ICMP redirect, ICMP_GEN and BROADCAST

class-map match-any system-cpp-police-stackwise-virt-control

description Stackwise Virtual

class-map match-any non-client-nrt-class

class-map match-any system-cpp-police-routing-control

description Routing control and Low Latency

class-map match-any system-cpp-police-protocol-snooping

description Protocol snooping

class-map match-any system-cpp-police-dhcp-snooping

description DHCP snooping

class-map match-any system-cpp-police-system-critical

description System Critical and Gold Pkt

!

policy-map system-cpp-policy

!

!

!

!

!

!

!

!

!

!

!

!

!

interface GigabitEthernet0/0

vrf forwarding Mgmt-vrf

no ip address

shutdown

negotiation auto

!

interface GigabitEthernet1/0/1

no switchport

ip address 10.0.3.2 255.255.255.252

ip ospf 1 area 0

ipv6 address 1003::2/64

ipv6 ospf 1 area 0

!

interface GigabitEthernet1/0/2

no switchport

ip address 10.0.2.2 255.255.255.252

ip ospf 1 area 0

ipv6 address 1002::2/64

ipv6 ospf 1 area 0

!

interface GigabitEthernet1/0/3

no switchport

ip address 10.9.1.1 255.255.255.252

ip ospf 1 area 9

ipv6 address 1091::1/64

ipv6 ospf 1 area 9

!

interface GigabitEthernet1/0/4

!

interface GigabitEthernet1/0/5

!

interface GigabitEthernet1/0/6

!

interface GigabitEthernet1/0/7

!

interface GigabitEthernet1/0/8

!

interface GigabitEthernet1/0/9

!

interface GigabitEthernet1/0/10

!

interface GigabitEthernet1/0/11

!

interface GigabitEthernet1/0/12

!

interface GigabitEthernet1/0/13

!

interface GigabitEthernet1/0/14

!

interface GigabitEthernet1/0/15

!

interface GigabitEthernet1/0/16

!

interface GigabitEthernet1/0/17

!

interface GigabitEthernet1/0/18

!

interface GigabitEthernet1/0/19

!

interface GigabitEthernet1/0/20

!

interface GigabitEthernet1/0/21

!

interface GigabitEthernet1/0/22

!

interface GigabitEthernet1/0/23

!

interface GigabitEthernet1/0/24

!

interface GigabitEthernet1/1/1

!

interface GigabitEthernet1/1/2

!

interface GigabitEthernet1/1/3

!

interface GigabitEthernet1/1/4

!

interface Vlan1

no ip address

shutdown

!

router ospf 1

network 10.9.1.0 0.0.0.3 area 9

!

ip forward-protocol nd

ip http server

ip http authentication local

ip http secure-server

!

!

ipv6 router ospf 1

!

!

!

control-plane

service-policy input system-cpp-policy

!

!

line con 0

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

line vty 5 15

login

!

!

!

!

!

!

!

End

S1 routes

S1#show ip route

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2

ia - IS-IS inter area, * - candidate default, U - per-user static route

o - ODR, P - periodic downloaded static route, H - NHRP, l - LISP

a - application route

+ - replicated route, % - next hop override, p - overrides from PfR

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 9 subnets, 2 masks

C        10.0.2.0/30 is directly connected, GigabitEthernet1/0/2

L        10.0.2.2/32 is directly connected, GigabitEthernet1/0/2

C        10.0.3.0/30 is directly connected, GigabitEthernet1/0/1

L        10.0.3.2/32 is directly connected, GigabitEthernet1/0/1

O        10.0.4.0/30 [110/2] via 10.0.3.1, 00:35:51, GigabitEthernet1/0/1

O IA     10.1.1.0/30 [110/3] via 10.0.3.1, 00:35:51, GigabitEthernet1/0/1

O IA     10.1.2.0/30 [110/4] via 10.0.3.1, 00:35:51, GigabitEthernet1/0/1

C        10.9.1.0/30 is directly connected, GigabitEthernet1/0/3

L        10.9.1.1/32 is directly connected, GigabitEthernet1/0/3

S1#show ipv6 route

IPv6 Routing Table - default - 10 entries

Codes: C - Connected, L - Local, S - Static, U - Per-user Static route

R - RIP, D - EIGRP, EX - EIGRP external, ND - ND Default

NDp - ND Prefix, DCE - Destination, NDr - Redirect, O - OSPF Intra

OI - OSPF Inter, OE1 - OSPF ext 1, OE2 - OSPF ext 2, ON1 - OSPF NSSA ext 1

ON2 - OSPF NSSA ext 2

C   1002::/64 [0/0]

via GigabitEthernet1/0/2, directly connected

L   1002::2/128 [0/0]

via GigabitEthernet1/0/2, receive

C   1003::/64 [0/0]

via GigabitEthernet1/0/1, directly connected

L   1003::2/128 [0/0]

via GigabitEthernet1/0/1, receive

O   1004::/64 [110/2]

via FE80::72D3:79FF:FE02:8161, GigabitEthernet1/0/1

OI  1011::/64 [110/3]

via FE80::72D3:79FF:FE02:8161, GigabitEthernet1/0/1

OI  1012::/64 [110/4]

via FE80::72D3:79FF:FE02:8161, GigabitEthernet1/0/1

C   1091::/64 [0/0]

via GigabitEthernet1/0/3, directly connected

L   1091::1/128 [0/0]

via GigabitEthernet1/0/3, receive

L   FF00::/8 [0/0]

via Null0, receive

S2 show run

Building configuration...

Current configuration : 8616 bytes

!

! Last configuration change at 21:37:15 UTC Mon Sep 15 2025

!

version 16.9

no service pad

service timestamps debug datetime msec

service timestamps log datetime msec

! Call-home is enabled by Smart-Licensing.

service call-home

no platform punt-keepalive disable-kernel-core

!

hostname S2

!

!

vrf definition Mgmt-vrf

!

address-family ipv4

exit-address-family

!

address-family ipv6

exit-address-family

!

!

!

!

!

no aaa new-model

switch 1 provision c9200l-24t-4g

!

!

!

!

!

!

!

!

!

ip routing

ip arp entry learn 10240

!

!

!

!

login on-success log

ipv6 unicast-routing

call-home

! If contact email address in call-home is configured as sch-smart-licensing@cisco.com

! the email address configured in Cisco Smart License Portal will be used as contact email address to send SCH notifications.

contact-email-addr sch-smart-licensing@cisco.com

profile "CiscoTAC-1"

active

destination transport-method http

no destination transport-method email

!

crypto pki trustpoint SLA-TrustPoint

enrollment pkcs12

revocation-check crl

!

crypto pki trustpoint TP-self-signed-2617307985

enrollment selfsigned

subject-name cn=IOS-Self-Signed-Certificate-2617307985

revocation-check none

rsakeypair TP-self-signed-2617307985

!

!

crypto pki certificate chain SLA-TrustPoint

certificate ca 01

30820321 30820209 A0030201 02020101 300D0609 2A864886 F70D0101 0B050030

32310E30 0C060355 040A1305 43697363 6F312030 1E060355 04031317 43697363

6F204C69 63656E73 696E6720 526F6F74 20434130 1E170D31 33303533 30313934

3834375A 170D3338 30353330 31393438 34375A30 32310E30 0C060355 040A1305

43697363 6F312030 1E060355 04031317 43697363 6F204C69 63656E73 696E6720

526F6F74 20434130 82012230 0D06092A 864886F7 0D010101 05000382 010F0030

82010A02 82010100 A6BCBD96 131E05F7 145EA72C 2CD686E6 17222EA1 F1EFF64D

CBB4C798 212AA147 C655D8D7 9471380D 8711441E 1AAF071A 9CAE6388 8A38E520

1C394D78 462EF239 C659F715 B98C0A59 5BBB5CBD 0CFEBEA3 700A8BF7 D8F256EE

4AA4E80D DB6FD1C9 60B1FD18 FFC69C96 6FA68957 A2617DE7 104FDC5F EA2956AC

7390A3EB 2B5436AD C847A2C5 DAB553EB 69A9A535 58E9F3E3 C0BD23CF 58BD7188

68E69491 20F320E7 948E71D7 AE3BCC84 F10684C7 4BC8E00F 539BA42B 42C68BB7

C7479096 B4CB2D62 EA2F505D C7B062A4 6811D95B E8250FC4 5D5D5FB8 8F27D191

C55F0D76 61F9A4CD 3D992327 A8BB03BD 4E6D7069 7CBADF8B DF5F4368 95135E44

DFC7C6CF 04DD7FD1 02030100 01A34230 40300E06 03551D0F 0101FF04 04030201

06300F06 03551D13 0101FF04 05300301 01FF301D 0603551D 0E041604 1449DC85

4B3D31E5 1B3E6A17 606AF333 3D3B4C73 E8300D06 092A8648 86F70D01 010B0500

03820101 00507F24 D3932A66 86025D9F E838AE5C 6D4DF6B0 49631C78 240DA905

604EDCDE FF4FED2B 77FC460E CD636FDB DD44681E 3A5673AB 9093D3B1 6C9E3D8B

D98987BF E40CBD9E 1AECA0C2 2189BB5C 8FA85686 CD98B646 5575B146 8DFC66A8

467A3DF4 4D565700 6ADF0F0D CF835015 3C04FF7C 21E878AC 11BA9CD2 55A9232C

7CA7B7E6 C1AF74F6 152E99B7 B1FCF9BB E973DE7F 5BDDEB86 C71E3B49 1765308B

5FB0DA06 B92AFE7F 494E8A9E 07B85737 F3A58BE1 1A48A229 C37C1E69 39F08678

80DDCD16 D6BACECA EEBC7CF9 8428787B 35202CDC 60E4616A B623CDBD 230E3AFB

418616A9 4093E049 4D10AB75 27E86F73 932E35B5 8862FDAE 0275156F 719BB2F0

D697DF7F 28

quit

crypto pki certificate chain TP-self-signed-2617307985

certificate self-signed 01

30820330 30820218 A0030201 02020101 300D0609 2A864886 F70D0101 05050030

31312F30 2D060355 04031326 494F532D 53656C66 2D536967 6E65642D 43657274

69666963 6174652D 32363137 33303739 3835301E 170D3235 30393135 31373531

30385A17 0D333030 31303130 30303030 305A3031 312F302D 06035504 03132649

4F532D53 656C662D 5369676E 65642D43 65727469 66696361 74652D32 36313733

30373938 35308201 22300D06 092A8648 86F70D01 01010500 0382010F 00308201

0A028201 01009E8B 48B178D1 A34ECC15 886A782B D6F7EA6F 773D4E29 0B8595D5

2B97D277 5A99CCFB D0B9C288 FC7934F8 8B01DAA1 7A88998A AD84C439 17317F65

D43BAAB8 6EE988CB D4D2410D BC23FDF8 CECF9619 E6BDEF41 C43B4395 67AEEA7A

8BFBA93A B17E984B FB12641B AD3588FE BAE41700 8EFBBAFD 220D8347 213021AD

0CC04CB9 9862CA1F 0EFF2FD7 BE17644F 419F3EE9 9879D71C 3ED002C2 7979C91D

0EDC4234 A5EF49B2 46481B92 B3926D47 D068BFE6 266510F1 D3836296 5BC15B3D

F1E70EF7 0E21479F 6800F7FA CB2517CF 55F514CB 797F5362 2F471C02 B943A07A

4ADEDC3D 1EE860F1 BC2CCD66 5E60934A BFEE8C04 E04706CD 0F451A5B 666F9AFD

97B61B7E 12B70203 010001A3 53305130 0F060355 1D130101 FF040530 030101FF

301F0603 551D2304 18301680 142FDCFF D966DAB2 C7F2F808 36941696 F8726816

2C301D06 03551D0E 04160414 2FDCFFD9 66DAB2C7 F2F80836 941696F8 7268162C

300D0609 2A864886 F70D0101 05050003 82010100 206948B9 9E044E74 B1F74315

199827BC F183D1E6 19A30435 E7DC5E0E FB07732C EB3A4800 7EF6842C 5FCAFD95

E1E30D31 640754FB E7AF6A6E C5A2DB81 7E5C675F 40F2A037 F365B4CC D796CBE6

3AC08218 D67D4A78 C941DCDF 9FC9F585 FE032900 B1DCC6DE 528EDC5C 71CAE474

80C01D0E 6DA52AFA CF526D64 FB9DEA50 EEF55189 9765185D 0A929615 9A85FEA9

D40DE576 F2576AEE 5DC3CAD7 7AF3DCE5 72083B5D F26131C8 21EFC539 795BBE35

FDD031BC 192E3E4A 1E0A8AD7 809CB946 C1C588B0 C41EA60C 69CA7DF6 220CAD34

D089C2DF 4A44E122 C7B519F0 70C4CFB9 680D129F 20EE9D9C FED47081 3CCFB5B4

7326A47C 7F082833 ECC935E2 8A91D63D 5437DE8B

quit

!

license boot level network-essentials addon dna-essentials

!

!

diagnostic bootup level minimal

!

spanning-tree mode rapid-pvst

spanning-tree extend system-id

!

!

!

redundancy

mode sso

!

!

!

class-map match-any system-cpp-police-topology-control

description Topology control

class-map match-any system-cpp-police-sw-forward

description Sw forwarding, L2 LVX data, LOGGING

class-map match-any system-cpp-default

description Inter FED, EWLC control, EWLC data

class-map match-any system-cpp-police-sys-data

description Learning cache ovfl, High Rate App, Exception, EGR Exception, NFL SAMPLED DATA, RPF Failed

class-map match-any system-cpp-police-punt-webauth

description Punt Webauth

class-map match-any system-cpp-police-l2lvx-control

description L2 LVX control packets

class-map match-any system-cpp-police-forus

description Forus Address resolution and Forus traffic

class-map match-any system-cpp-police-multicast-end-station

description MCAST END STATION

class-map match-any system-cpp-police-multicast

description Transit Traffic and MCAST Data

class-map match-any system-cpp-police-l2-control

description L2 control

class-map match-any system-cpp-police-dot1x-auth

description DOT1X Auth

class-map match-any system-cpp-police-data

description ICMP redirect, ICMP_GEN and BROADCAST

class-map match-any system-cpp-police-stackwise-virt-control

description Stackwise Virtual

class-map match-any non-client-nrt-class

class-map match-any system-cpp-police-routing-control

description Routing control and Low Latency

class-map match-any system-cpp-police-protocol-snooping

description Protocol snooping

class-map match-any system-cpp-police-dhcp-snooping

description DHCP snooping

class-map match-any system-cpp-police-system-critical

description System Critical and Gold Pkt

!

policy-map system-cpp-policy

!

!

!

!

!

!

!

!

!

!

!

!

!

interface GigabitEthernet0/0

vrf forwarding Mgmt-vrf

no ip address

shutdown

negotiation auto

!

interface GigabitEthernet1/0/1

no switchport

ip address 10.9.1.2 255.255.255.252

ip ospf 1 area 9

ipv6 address 1091::2/64

ipv6 ospf 1 area 9

!

interface GigabitEthernet1/0/2

!

interface GigabitEthernet1/0/3

!

interface GigabitEthernet1/0/4

!

interface GigabitEthernet1/0/5

!

interface GigabitEthernet1/0/6

!

interface GigabitEthernet1/0/7

!

interface GigabitEthernet1/0/8

!

interface GigabitEthernet1/0/9

!

interface GigabitEthernet1/0/10

!

interface GigabitEthernet1/0/11

!

interface GigabitEthernet1/0/12

!

interface GigabitEthernet1/0/13

!

interface GigabitEthernet1/0/14

!

interface GigabitEthernet1/0/15

!

interface GigabitEthernet1/0/16

!

interface GigabitEthernet1/0/17

!

interface GigabitEthernet1/0/18

!

interface GigabitEthernet1/0/19

!

interface GigabitEthernet1/0/20

!

interface GigabitEthernet1/0/21

!

interface GigabitEthernet1/0/22

!

interface GigabitEthernet1/0/23

!

interface GigabitEthernet1/0/24

!

interface GigabitEthernet1/1/1

!

interface GigabitEthernet1/1/2

!

interface GigabitEthernet1/1/3

!

interface GigabitEthernet1/1/4

!

interface Vlan1

no ip address

shutdown

!

router ospfv3 1

!

address-family ipv6 unicast

exit-address-family

!

router ospf 1

!

ip forward-protocol nd

ip http server

ip http authentication local

ip http secure-server

!

!

!

!

control-plane

service-policy input system-cpp-policy

!

!

line con 0

stopbits 1

line aux 0

stopbits 1

line vty 0 4

login

line vty 5 15

login

!

!

!

!

!

!

!

End

S2 show route

S2#show ip route

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2

ia - IS-IS inter area, * - candidate default, U - per-user static route

o - ODR, P - periodic downloaded static route, H - NHRP, l - LISP

a - application route

+ - replicated route, % - next hop override, p - overrides from PfR

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 7 subnets, 2 masks

O IA     10.0.2.0/30 [110/2] via 10.9.1.1, 00:36:30, GigabitEthernet1/0/1

O IA     10.0.3.0/30 [110/2] via 10.9.1.1, 00:36:30, GigabitEthernet1/0/1

O IA     10.0.4.0/30 [110/3] via 10.9.1.1, 00:36:22, GigabitEthernet1/0/1

O IA     10.1.1.0/30 [110/4] via 10.9.1.1, 00:36:22, GigabitEthernet1/0/1

O IA     10.1.2.0/30 [110/5] via 10.9.1.1, 00:36:22, GigabitEthernet1/0/1

C        10.9.1.0/30 is directly connected, GigabitEthernet1/0/1

L        10.9.1.2/32 is directly connected, GigabitEthernet1/0/1

S2#show ipv6 route

IPv6 Routing Table - default - 8 entries

Codes: C - Connected, L - Local, S - Static, U - Per-user Static route

R - RIP, D - EIGRP, EX - EIGRP external, ND - ND Default

NDp - ND Prefix, DCE - Destination, NDr - Redirect, O - OSPF Intra

OI - OSPF Inter, OE1 - OSPF ext 1, OE2 - OSPF ext 2, ON1 - OSPF NSSA ext 1

ON2 - OSPF NSSA ext 2

OI  1002::/64 [110/2]

via FE80::C64D:84FF:FE2B:E5D8, GigabitEthernet1/0/1

OI  1003::/64 [110/2]

via FE80::C64D:84FF:FE2B:E5D8, GigabitEthernet1/0/1

OI  1004::/64 [110/3]

via FE80::C64D:84FF:FE2B:E5D8, GigabitEthernet1/0/1

OI  1011::/64 [110/4]

via FE80::C64D:84FF:FE2B:E5D8, GigabitEthernet1/0/1

OI  1012::/64 [110/5]

via FE80::C64D:84FF:FE2B:E5D8, GigabitEthernet1/0/1

C   1091::/64 [0/0]

via GigabitEthernet1/0/1, directly connected

L   1091::2/128 [0/0]

via GigabitEthernet1/0/1, receive

L   FF00::/8 [0/0]

via Null0, receive

R4 areas

R4#show ospf 1

OSPFv3 1 address-family ipv6

Router ID 10.0.4.2

Supports NSSA (compatible with RFC 3101)

Supports Database Exchange Summary List Optimization (RFC 5243)

Event-log enabled, Maximum number of events: 1000, Mode: cyclic

It is an area border router

Router is not originating router-LSAs with maximum metric

Initial SPF schedule delay 50 msecs

Minimum hold time between two consecutive SPFs 200 msecs

Maximum wait time between two consecutive SPFs 5000 msecs

Initial LSA throttle delay 50 msecs

Minimum hold time for LSA throttle 200 msecs

Maximum wait time for LSA throttle 5000 msecs

Minimum LSA arrival 100 msecs

LSA group pacing timer 240 secs

Interface flood pacing timer 33 msecs

Retransmission pacing timer 66 msecs

Retransmission limit dc 24 non-dc 24

EXCHANGE/LOADING adjacency limit: initial 300, process maximum 300

Number of external LSA 0. Checksum Sum 0x000000

Number of areas in this router is 2. 2 normal 0 stub 0 nssa

Graceful restart helper support enabled

Reference bandwidth unit is 100 mbps

RFC1583 compatibility enabled

Area BACKBONE(0)

Number of interfaces in this area is 1

SPF algorithm executed 26 times

Number of LSA 15. Checksum Sum 0x079082

Number of DCbitless LSA 0

Number of indication LSA 0

Number of DoNotAge LSA 0

Flood list length 0

Area 1

Number of interfaces in this area is 1

SPF algorithm executed 3 times

Number of LSA 13. Checksum Sum 0x0784F7

Number of DCbitless LSA 0

Number of indication LSA 0

Number of DoNotAge LSA 0

Flood list length 0

S1 areas

S1#show ospf 1

OSPFv3 1 address-family ipv6

Router ID 10.0.3.2

Supports NSSA (compatible with RFC 3101)

Supports Database Exchange Summary List Optimization (RFC 5243)

Event-log enabled, Maximum number of events: 1000, Mode: cyclic

It is an area border router

Router is not originating router-LSAs with maximum metric

Initial SPF schedule delay 50 msecs

Minimum hold time between two consecutive SPFs 200 msecs

Maximum wait time between two consecutive SPFs 5000 msecs

Initial LSA throttle delay 50 msecs

Minimum hold time for LSA throttle 200 msecs

Maximum wait time for LSA throttle 5000 msecs

Minimum LSA arrival 100 msecs

LSA group pacing timer 240 secs

Interface flood pacing timer 33 msecs

Retransmission pacing timer 66 msecs

Retransmission limit dc 24 non-dc 24

EXCHANGE/LOADING adjacency limit: initial 300, process maximum 300

Number of external LSA 0. Checksum Sum 0x000000

Number of areas in this router is 2. 2 normal 0 stub 0 nssa

Graceful restart helper support enabled

Reference bandwidth unit is 100 mbps

RFC1583 compatibility enabled

Area BACKBONE(0)

Number of interfaces in this area is 2

SPF algorithm executed 15 times

Number of LSA 19. Checksum Sum 0x08EDE7

Number of DCbitless LSA 0

Number of indication LSA 0

Number of DoNotAge LSA 0

Flood list length 0

Area 9

Number of interfaces in this area is 1

SPF algorithm executed 5 times

Number of LSA 11. Checksum Sum 0x05704F

Number of DCbitless LSA 0

Number of indication LSA 0

Number of DoNotAge LSA 0

Flood list length 0

Problems

An issue we had was our traceroute not going all the way across the network. We saw where the packet got stuck and went into the show run on that device. It was particularly challenging navigating the maze of information displayed by the show run, but eventually we found the problem – there was no ipv6 ospf 1 area 0 command configured on one of the router interfaces.

Conclusion

In short, we configured multiarea OSPF by issuing some new commands. While we ran into some problems near the end of the lab, we were able to quickly locate the issue and solve the problem.